CCP-CN-S2026 - Problem 2: ISP Network Failover

Week 2: Implementation, Connectivity, VLAN Note

Static Route Based ISP Network Failover

Implementation Note: GNS3 setup was completed, but the required Cisco IOS image was not available. Therefore, the same ISP failover design was implemented and validated in Cisco Packet Tracer using 3 routers and 2 PCs.

# 1. Implementation Summary

The topology was implemented in Cisco Packet Tracer with 3 Cisco 2911 routers and 2 PCs. The selected Problem 2 does not require VLANs; therefore, VLAN screenshots are not applicable.

Final topology with all links green.

# 2. Router Configuration Summary

## City-A-Core

G0/0 = 192.168.10.1/24

G0/1 = 10.0.12.1/30

G0/2 = 10.0.13.1/30

Primary route to 192.168.20.0/24 via 10.0.12.2

Floating backup route to 192.168.20.0/24 via 10.0.13.2 AD 100

## City-B

G0/0 = 10.0.12.2/30

G0/1 = 10.0.23.2/30

G0/2 = 192.168.20.1/24

Primary return route to 192.168.10.0/24 via 10.0.12.1

Floating backup return route via 10.0.23.1 AD 100

## City-C-Backup

G0/0 = 10.0.13.2/30

G0/1 = 10.0.23.1/30

Route to Customer-A LAN via 10.0.13.1

Route to Customer-B LAN via 10.0.23.2

# 3. Connectivity Tests

| Test | Command/Evidence | Expected Result | Status |
| --- | --- | --- | --- |
| Normal ping before failure | normal-ping-before-failure.png | Customer-A reaches Customer-B | Pass |
| Routing table before failure | routing-table-before-failure.png | Route to 192.168.20.0/24 via 10.0.12.2 with AD 1 | Pass |

Customer-A successfully pinged Customer-B before failure.

City-A-Core routing table before link failure. It shows primary static route via 10.0.12.2.

# 4. Running Config Evidence

Attach City-A-Core running-config.

Attach City-B running-config.

Attach City-C-Backup running-config.

# 5. Week 2 Status Report

Topology implemented.

Router interfaces configured.

Static and floating static routes configured.

Normal connectivity verified.

VLANs are not applicable for Problem 2.