CCP-CN-S2026 - Problem 2: ISP Network Failover

Week 1: Topology, IP Plan, GitHub

Static Route Based ISP Network Failover

Implementation Note: GNS3 setup was completed, but the required Cisco IOS image was not available. Therefore, the same ISP failover design was implemented and validated in Cisco Packet Tracer using 3 routers and 2 PCs.

# 1. Problem Selection

Selected Problem: Problem 2 - ISP Network. The project designs a three-city ISP topology with a primary route and a backup route that activates when the primary link fails.

# 2. Team Charter

| Member | Combined Role | Responsibilities |
| --- | --- | --- |
| Member 1 | Network Architect + Testing Lead | Topology design, IP addressing, router configuration, ping tests, route verification |
| Member 1 | Documentation + Report Lead | Weekly reports, final report, evidence organization, screenshots placement |
| Member 1 | Project Manager + GitHub/Submission Lead | Repository/folder structure, file naming, submission checklist, video coordination |

# 3. Logical Topology

Customer-A connects to City-A-Core.

City-A-Core connects directly to City-B through the primary link.

City-A-Core connects to City-C-Backup through the backup link.

City-C-Backup connects to City-B to complete the backup path.

Customer-B connects to City-B.

Primary Path: Customer-A -> City-A-Core -> City-B -> Customer-B

Backup Path: Customer-A -> City-A-Core -> City-C-Backup -> City-B -> Customer-B

Use this as the final topology evidence after all links are restored and green.

# 4. IP Addressing Plan

| Network/Link | Device | Interface | IP Address | Mask | Gateway/Next Hop | Purpose |
| --- | --- | --- | --- | --- | --- | --- |
| Customer-A LAN | Customer-A PC | NIC | 192.168.10.10 | 255.255.255.0 | 192.168.10.1 | Source PC |
| Customer-A LAN | City-A-Core | G0/0 | 192.168.10.1 | 255.255.255.0 | - | Gateway |
| Primary Link | City-A-Core | G0/1 | 10.0.12.1 | 255.255.255.252 | 10.0.12.2 | Primary path |
| Primary Link | City-B | G0/0 | 10.0.12.2 | 255.255.255.252 | 10.0.12.1 | Primary path |
| Backup Link 1 | City-A-Core | G0/2 | 10.0.13.1 | 255.255.255.252 | 10.0.13.2 | Backup path |
| Backup Link 1 | City-C-Backup | G0/0 | 10.0.13.2 | 255.255.255.252 | 10.0.13.1 | Backup path |
| Backup Link 2 | City-C-Backup | G0/1 | 10.0.23.1 | 255.255.255.252 | 10.0.23.2 | Backup path |
| Backup Link 2 | City-B | G0/1 | 10.0.23.2 | 255.255.255.252 | 10.0.23.1 | Backup path |
| Customer-B LAN | City-B | G0/2 | 192.168.20.1 | 255.255.255.0 | - | Gateway |
| Customer-B LAN | Customer-B PC | NIC | 192.168.20.10 | 255.255.255.0 | 192.168.20.1 | Destination PC |

# 5. GitHub Repository name: 

CCP_ISP_Failover

https://github.com/Ahmed-sys-coder/CCP_ISP_Failover

# 6. Risk Assessment Matrix

| Risk | Impact | Probability | Mitigation |
| --- | --- | --- | --- |
| Cisco IOS image unavailable in GNS3 | High | High | Equivalent implementation completed in Cisco Packet Tracer |
| Interface naming mismatch | Medium | Medium | Verified interfaces using show ip interface brief |
| Backup route not activating | High | Medium | Used floating static route with AD 100 |
| Missing screenshots | Medium | Low | Captured normal, failure, backup, and restore evidence |

# 7. Week 1 Status Report

Problem selected and understood.

Topology and IP plan prepared.

Roles assigned for a member group.

Risk matrix completed.

Practical implementation planned for Packet Tracer because Cisco IOS image was not available for GNS3.