CCP-CN-S2026 - Problem 2: ISP Network Failover

Week 3: Security, VPN, L2 Security Note + Failure Simulation

Static Route Based ISP Network Failover

Implementation Note: GNS3 setup was completed, but the required Cisco IOS image was not available. Therefore, the same ISP failover design was implemented and validated in Cisco Packet Tracer using 3 routers and 2 PCs.

# 1. Requirement Mapping

The LMS folder title mentions security, VPN, and L2 security. However, the selected Problem 2 is an ISP failover problem. It requires static routes, different administrative distances, and failure testing. VPN and L2 security are not part of this selected problem.

# 2. Controlled Failure Test

To simulate a primary link failure, the City-A-Core interface connected to City-B was shut down.

| Device | Interface | Action | Purpose |
| --- | --- | --- | --- |
| City-A-Core | GigabitEthernet0/1 | shutdown | Simulate primary City-A to City-B link failure |

# 3. Commands Used

enable

configure terminal

interface gigabitEthernet0/1

shutdown

end

show ip route

# 4. Expected and Actual Result

| State | Route Expected | Evidence |
| --- | --- | --- |
| After primary failure | S 192.168.20.0/24 [100/0] via 10.0.13.2 | after-failure-routing-table.png |
| Backup connectivity | Customer-A ping to 192.168.20.10 succeeds | backup-path-ping-success.png |

Topology evidence showing the primary link failure state.

City-A-Core routing table after primary link failure. Backup route via 10.0.13.2 becomes active.

Ping success through backup path after primary failure.

# 5. Week 3 Status Report

Primary link failure simulated.

Backup route activated.

Ping through backup path succeeded.

Security/VPN/L2 items marked not applicable for selected Problem 2.