CCP-CN-S2026 Final Project Report

Problem 2: ISP Network Failover

Static Route Based ISP Network Failover using Cisco Packet Tracer

Course: Computer Networks
Project Type: Complex Computing Problem (CCP)
Implementation Tool: Cisco Packet Tracer

| Group Member | Role |
| --- | --- |
| Member | Network Architect + Testing Lead |
| Member | Documentation + Report Lead |
| Member | Project Manager + GitHub/Submission Lead |

Submission: Week 6 Final Report, Video and Presentation

# Abstract

This report presents the design, implementation, and validation of a three-city ISP network failover solution. The selected problem is Problem 2: ISP Network. The network uses a primary direct path between City-A-Core and City-B and a backup transit path through City-C-Backup. Static routes and floating static routes with different administrative distances are used to achieve automatic path selection during normal operation, primary link failure, and primary link restoration. The practical implementation was validated in Cisco Packet Tracer using 3 Cisco 2911 routers and 2 PCs.

# Implementation Note

GNS3 Desktop and GNS3 VM setup were completed, but the required Cisco IOS image for the Cisco 7200 appliance was unavailable. Therefore, the same ISP failover design was implemented and validated in Cisco Packet Tracer. The topology, IP addressing, static routes, administrative distance behavior, failover testing, and evidence screenshots remain aligned with the selected ISP failover problem.

# Table of Contents

1. Introduction

2. Problem Scenario

3. Project Objectives

4. Tools and Technologies

5. Network Topology Design

6. IP Addressing Plan

7. Routing Design and Configuration

8. Testing Methodology

9. Results and Evidence

10. Failure Response and Mitigation

11. Limitations

12. Conclusion

Appendix A: Router Configuration Summary

# 1. Introduction

Internet Service Provider networks require reliable connectivity even when a primary link fails. This project demonstrates a simple but effective ISP resilience design using static routes and floating static routes. The solution ensures that traffic between Customer-A and Customer-B normally follows the direct City-A-Core to City-B path, and automatically switches to a backup path through City-C-Backup when the direct link fails.

# 2. Problem Scenario

ConnectSphere ISP has three cities: City A, City B, and City C. City B customers experience connectivity loss when the direct primary link fails. The ISP requires a backup route so customer traffic can continue during the failure. The selected design uses City-C-Backup as an alternate transit router.

| City / Node | Role in Network |
| --- | --- |
| Customer-A | Source end-user PC in the 192.168.10.0/24 LAN |
| City-A-Core | Core router and primary source-side gateway |
| City-B | Customer-B side router and destination gateway |
| City-C-Backup | Backup transit router used during primary link failure |
| Customer-B | Destination end-user PC in the 192.168.20.0/24 LAN |

# 3. Project Objectives

Design a three-router ISP topology with two customer LANs.

Configure a direct primary path from City-A-Core to City-B.

Configure a backup transit path through City-C-Backup.

Use floating static routes with a higher administrative distance for failover.

Validate normal connectivity, failure state, backup path operation, and primary route restoration.

Capture evidence screenshots for final submission.

# 4. Tools and Technologies

| Tool / Technology | Purpose |
| --- | --- |
| Cisco Packet Tracer | Network topology implementation and simulation |
| Cisco 2911 Routers | Routing between ISP city networks |
| PC-PT End Devices | Customer-A and Customer-B connectivity testing |
| Static Routing | Manual route control and path selection |
| Floating Static Routes | Backup routes using higher administrative distance |
| Ping | End-to-end reachability verification |
| show ip route | Routing table and failover verification |

# 5. Network Topology Design

The topology contains two customer PCs and three routers. The primary path is Customer-A -> City-A-Core -> City-B -> Customer-B. The backup path is Customer-A -> City-A-Core -> City-C-Backup -> City-B -> Customer-B.

Figure 1: Final topology with all links restored and green.

# 6. IP Addressing Plan

| Network | Device | Interface | IP Address | Subnet Mask | Gateway / Next Hop |
| --- | --- | --- | --- | --- | --- |
| Customer-A LAN | Customer-A PC | NIC | 192.168.10.10 | 255.255.255.0 | 192.168.10.1 |
| Customer-A LAN | City-A-Core | G0/0 | 192.168.10.1 | 255.255.255.0 | - |
| Primary Link | City-A-Core | G0/1 | 10.0.12.1 | 255.255.255.252 | 10.0.12.2 |
| Primary Link | City-B | G0/0 | 10.0.12.2 | 255.255.255.252 | 10.0.12.1 |
| Backup Link 1 | City-A-Core | G0/2 | 10.0.13.1 | 255.255.255.252 | 10.0.13.2 |
| Backup Link 1 | City-C-Backup | G0/0 | 10.0.13.2 | 255.255.255.252 | 10.0.13.1 |
| Backup Link 2 | City-C-Backup | G0/1 | 10.0.23.1 | 255.255.255.252 | 10.0.23.2 |
| Backup Link 2 | City-B | G0/1 | 10.0.23.2 | 255.255.255.252 | 10.0.23.1 |
| Customer-B LAN | City-B | G0/2 | 192.168.20.1 | 255.255.255.0 | - |
| Customer-B LAN | Customer-B PC | NIC | 192.168.20.10 | 255.255.255.0 | 192.168.20.1 |

# 7. Routing Design and Configuration

The routing design uses a primary static route and a backup floating static route. The primary route uses the default administrative distance of 1, while the backup route uses administrative distance 100. This makes the primary path preferred during normal operation and the backup path active only when the primary route becomes unavailable.

| Router | Route Purpose | Command Summary |
| --- | --- | --- |
| City-A-Core | Primary route to Customer-B LAN | ip route 192.168.20.0 255.255.255.0 10.0.12.2 |
| City-A-Core | Backup route to Customer-B LAN | ip route 192.168.20.0 255.255.255.0 10.0.13.2 100 |
| City-B | Primary return route to Customer-A LAN | ip route 192.168.10.0 255.255.255.0 10.0.12.1 |
| City-B | Backup return route to Customer-A LAN | ip route 192.168.10.0 255.255.255.0 10.0.23.1 100 |
| City-C-Backup | Route to Customer-A LAN | ip route 192.168.10.0 255.255.255.0 10.0.13.1 |
| City-C-Backup | Route to Customer-B LAN | ip route 192.168.20.0 255.255.255.0 10.0.23.2 |

# 8. Testing Methodology

Testing was performed in four major stages: normal operation, primary link failure, backup path verification, and primary link restoration.

| Test ID | Test Description | Command / Evidence | Expected Result |
| --- | --- | --- | --- |
| T1 | Normal ping before failure | ping 192.168.20.10 | Customer-A reaches Customer-B |
| T2 | Route table before failure | show ip route on City-A-Core | Route via 10.0.12.2 with AD 1 |
| T3 | Primary link shutdown | shutdown on G0/1 of City-A-Core | Primary link goes down |
| T4 | Route table after failure | show ip route on City-A-Core | Route via 10.0.13.2 with AD 100 |
| T5 | Backup path ping | ping 192.168.20.10 | Ping succeeds through City-C-Backup |
| T6 | Primary link restore | no shutdown on G0/1 | Primary link restored |
| T7 | Final route and ping | show ip route and ping | Route returns to 10.0.12.2 and ping succeeds |

# 9. Results and Evidence

## 9.1 Normal Connectivity Before Failure

Customer-A successfully pinged Customer-B during normal operation. Initial timeouts can occur due to ARP resolution, but replies confirm successful connectivity.

Figure 2: Normal ping before failure.

The routing table before failure shows the primary route to 192.168.20.0/24 through 10.0.12.2 with administrative distance 1.

Figure 3: Routing table before failure - primary route via 10.0.12.2.

## 9.2 Primary Link Failure

The primary link between City-A-Core and City-B was administratively shut down to simulate link failure.

Figure 4: Primary link failure topology.

After failure, the routing table shows the floating static route through City-C-Backup with administrative distance 100.

Figure 5: Routing table after failure - backup route via 10.0.13.2.

## 9.3 Backup Path Verification

After primary link failure, Customer-A was still able to reach Customer-B through the backup path. The increased TTL value confirms route behavior changed compared to the normal path.

Figure 6: Backup path ping success.

## 9.4 Primary Link Restoration

After the primary interface was restored using no shutdown, the route table again preferred the primary path via 10.0.12.2 with administrative distance 1.

Figure 7: Routing table after primary link restore.

The final ping after restore confirms that end-to-end connectivity remained successful.

Figure 8: Final ping after primary route restoration.

# 10. Failure Response and Mitigation

| Stage | Observation | Response / Mitigation |
| --- | --- | --- |
| Normal State | Route via 10.0.12.2 is active. | Traffic uses direct City-A-Core to City-B link. |
| Failure State | Primary link is down and route via 10.0.12.2 is unavailable. | Floating static route via 10.0.13.2 becomes active. |
| Backup Operation | Ping succeeds through backup path. | City-C-Backup maintains service continuity. |
| Restoration | Primary route returns after no shutdown. | Traffic again uses the preferred direct route. |

# 11. Limitations

GNS3 Cisco IOS image was unavailable, so implementation was completed in Cisco Packet Tracer.

QoS, VPN, L2 security, and attack tools were not applicable to the selected Problem 2 scenario.

Monitoring was performed using ping and routing table verification, which are suitable for this static-route failover problem.

# 12. Conclusion

The project successfully demonstrates static route based ISP failover. During normal operation, Customer-A traffic to Customer-B uses the direct primary route via City-B. When the primary link fails, the router automatically selects the floating static backup route through City-C-Backup. After primary link restoration, the network returns to the preferred primary path. The evidence screenshots confirm successful connectivity, route change, backup operation, and recovery.

# Appendix A: Router Configuration Summary

City-A-Core Configuration Summary

interface g0/0
 ip address 192.168.10.1 255.255.255.0
 no shutdown
interface g0/1
 ip address 10.0.12.1 255.255.255.252
 no shutdown
interface g0/2
 ip address 10.0.13.1 255.255.255.252
 no shutdown
ip route 192.168.20.0 255.255.255.0 10.0.12.2
ip route 192.168.20.0 255.255.255.0 10.0.13.2 100

City-B Configuration Summary

interface g0/0
 ip address 10.0.12.2 255.255.255.252
 no shutdown
interface g0/1
 ip address 10.0.23.2 255.255.255.252
 no shutdown
interface g0/2
 ip address 192.168.20.1 255.255.255.0
 no shutdown
ip route 192.168.10.0 255.255.255.0 10.0.12.1
ip route 192.168.10.0 255.255.255.0 10.0.23.1 100

City-C-Backup Configuration Summary

interface g0/0
 ip address 10.0.13.2 255.255.255.252
 no shutdown
interface g0/1
 ip address 10.0.23.1 255.255.255.252
 no shutdown
ip route 192.168.10.0 255.255.255.0 10.0.13.1
ip route 192.168.20.0 255.255.255.0 10.0.23.2

Week 6 Deliverable Addendum

Final Report, Video Demonstration Script, Presentation Outline and Submission Checklist

# 1. Final Report

The complete final project report has been included above. It covers the problem scenario, objectives, tools, topology design, IP addressing plan, routing configuration, testing methodology, results, failure response, limitations, conclusion, and router configuration summary.

# 2. Video Demonstration Script

1. Introduce the project: Static Route Based ISP Network Failover for CCP-CN-S2026, Problem 2: ISP Network.

2. Show the final Cisco Packet Tracer topology with Customer-A, City-A-Core, City-B, City-C-Backup, and Customer-B.

3. Explain the primary path: Customer-A -> City-A-Core -> City-B -> Customer-B.

4. Explain the backup path: Customer-A -> City-A-Core -> City-C-Backup -> City-B -> Customer-B.

5. Show Customer-A pinging Customer-B before failure using ping 192.168.20.10.

6. Show the City-A-Core routing table before failure using show ip route. Explain that the route is via 10.0.12.2 with administrative distance 1.

7. Simulate failure by shutting down City-A-Core GigabitEthernet0/1.

8. Show the routing table after failure. Explain that the backup floating static route is active via 10.0.13.2 with administrative distance 100.

9. Show backup path ping success from Customer-A to Customer-B.

10. Restore the primary interface using no shutdown.

11. Show the route after restore and the final ping after restore.

12. Conclude that failover and recovery were successful.

# 3. Presentation Outline

| Slide | Title | Main Content |
| --- | --- | --- |
| 1 | Title | Static Route Based ISP Network Failover |
| 2 | Problem Scenario | Three-city ISP network with primary link failure issue |
| 3 | Objectives | Primary path, backup path, failover, and restore |
| 4 | Topology | Show final topology with all links restored and green |
| 5 | IP Addressing | Show Customer-A LAN, Customer-B LAN, primary link, and backup links |
| 6 | Routing Design | Static route AD 1 and floating static route AD 100 |
| 7 | Normal Operation | Show normal ping and routing table before failure |
| 8 | Failure Simulation | Show primary interface shutdown |
| 9 | Backup Result | Show backup route via 10.0.13.2 and backup ping success |
| 10 | Conclusion | Failover achieved successfully and primary path restored |

# 4. Week 6 Final Submission Checklist

- Video Link
https://drive.google.com/file/d/1wUaU75PZd0k4XRkTiDU9_LcpWfCz32uF/view?usp=sharing

- PPT Presentation Link

- https://docs.google.com/presentation/d/1A4JUvWLv1qdA_UxnblqtYyc74-GevidO/edit?usp=sharing&ouid=111565301925292724739&rtpof=true&sd=true

- Github Link

- https://github.com/Ahmed-sys-coder/CCP_ISP_Failover